| Course Code/Title: Computer Organization & Assembly Langua | Assessment Activity: LAB-6 | Batch/Semester: Batch(2023) Semester VI |
| --- | --- | --- |
| Dated:2/19/2026 | OBE- Target: | Weight of Marks: |
| Student ID: 4-38/2023/004 Student Name: Shahnil Sharma | Teacher Name: Ashar Ahmed | Score: |

## Objectives

• Perform subtraction using SUB instruction in 8086 Assembly.
• Understand difference between 8-bit registers (AL, BL) and 16-bit registers (AX, BX).
• Understand borrow concept using Carry Flag (CF).
• Observe register changes after subtraction.

## Theory

The SUB instruction subtracts source operand from destination operand. In 8-bit operations, registers like AL and BL are used. In 16-bit operations, AX and BX are used. If subtraction requires borrow, the Carry Flag (CF) becomes 1.

## Part A: 8-bit Subtraction

.model small
.stack 100h
.code
main:
    MOV AL, 09H
    MOV BL, 05H
    SUB AL, BL

    MOV AH, 4CH
    INT 21H
end main

Expected Result: AL = 04H, CF = 0

Step 1 execute 
MOV AL, 09H

Registers panel
AL = 09

Step 2 execute 
MOV BL, 05H

Registers Panel
: BL = 05

Step 3 execute
SUB AL, BL

Expected result:

09H - 05H = 04H

Registers Panel

AL = 04

Flags panel

CF = 0 :because no  borrow

## Part B: 16-bit Subtraction

.model small
.stack 100h
.code
main:
    MOV AX, 0005H
    MOV BX, 0008H
    SUB AX, BX

    MOV AH, 4CH
    INT 21H
end main

Expected Result: AX = FFFDH (0005H - 0008H = -3 in 16-bit form), CF = 1

Step 1
MOV AX, 0005H

AX Shows
AH = 00
AL = 05

Step 2
MOV BX, 0008H

BX Shows
BH = 00
BL = 08

Step 3
SUB AX, BX

Calculation:

0005H - 0008H = -3

AX = FFFD

AH = FF
AL = FD

Flags panel:

CF = 1 :this means Borrow executed

## Observation

In 8-bit subtraction, result is stored in AL and no borrow occurs if destination is larger. In 16-bit subtraction, when subtracting a larger number from a smaller one, borrow occurs and Carry Flag becomes 1.

## Conclusion

The lab demonstrates subtraction using both 8-bit and 16-bit registers. It also shows how the Carry Flag indicates borrow during subtraction.