| Course Code/Title: Computer Organization & Assembly Langua | Assessment Activity: LAB-7 | Batch/Semester: Batch(2023) Semester VI |
| --- | --- | --- |
| Dated:2/19/2026 | OBE- Target: | Weight of Marks: |
| Student ID: 4-38/2023/004 Student Name: Shahnil Sharma | Teacher Name: Ashar Ahmed | Score: |

## Objectives

• Design and implement addition and subtraction using 8086 Assembly.
• Apply concepts of 8-bit (AL, BL) and 16-bit (AX, BX) registers.
• Analyze Carry Flag (CF) and Zero Flag (ZF).
• Modify and extend the program logically.

## Theory

The 8086 processor performs arithmetic operations using instructions such as ADD and SUB. 8-bit operations use registers like AL and BL, while 16-bit operations use AX and BX. Carry Flag (CF) indicates overflow or borrow in unsigned operations, and Zero Flag (ZF) becomes 1 if the result of an operation is zero.

## Part A: 8-bit Arithmetic Program

.model small
.stack 100h
.code
main:
    MOV AL, 08H
    MOV BL, 04H
    ADD AL, BL

    SUB AL, BL

    MOV AH, 4CH
    INT 21H
end main

## Part B: 16-bit Arithmetic Program

.model small
.stack 100h
.code
main:
    MOV AX, 0005H
    MOV BX, 0005H
    ADD AX, BX

    SUB AX, BX

    MOV AH, 4CH
    INT 21H
end main

PF:FLAG=0

## Observation

Carry Flag becomes 1 if overflow or borrow occurs. Zero Flag becomes 1 when the result of an operation is zero.

## Conclusion

This lab demonstrates arithmetic operations using both 8-bit and 16-bit registers and shows how Carry and Zero flags help analyze results.